SSTP-VPN.

Secure Socket Tunneling Protocol (STP): It is a VPN protocol and was introduced by Microsoft windows vista. The TCP connection was established to an SSTP server over TCP port 443.

How it works? : SSTP works by establishing a secure connection between SSTP server and SSTP client, basically a secure tunnel was created between server and the client and all the data passing through the tunnel is encrypted.

Configuration: For this practical we need DC, member server, and client machine, we will configure our member server as a SSTP VPN server, in member server we need two adapters one connecting to our DC IP-range, and the other will connect to our client machine using a public IP-address.

First thing we have to download ‘Remote access’ in our member server, install the ‘Remote access’ , second we need to create a self-signed certificate, for that go to tools and click on IIS (Internet Information Service), click on your system name and double click on ‘server certificate’ icon, and navigate to the right hand side of the panel and click on ‘self-signed certificate’, give a relative name e.g. ‘For SSTP’, and hit ok, certificate will be created, now we have to keep it in to our C drive or any drive you want, for that double click on the certificate and navigate to the ‘details’ tab click on details tab, next, and browse it to the C drive, for confirmation open ‘This PC’ and go to C drive make a folder with the name ‘Certificate’ an move the certificate inside the folder, now share the folder with the read and write permission to everyone.

Now come back to the ‘Routing and Remote Access’ console wizard and right click on your system name and click on ‘Configure routing and remote access’, navigate to the ‘security tab’ and at the bottom of the page ‘certification’ drop down select your certificate, notice the name which will be like e.g. ‘sys.john.com’ etc, and navigate to the ‘ipv4’ tab select the ‘static IP’ radio button and Add a range of IP-address with the same subnet of our DC, and click on apply, it will take some time, after that hit ok, our SSTP configuration for member server is done.

Now go to your DC machine select any user and right click on the user and click on properties and navigate to the ‘dailin’ tab and select the ‘allow access’ radio button and hit enter.

Go to your client machine, to perform SSTP we need a copy of the certificate which we already kept it in the member server C drive, to do that we have to join our client machine to our domain, change the IP-address to the same range of our DC machine to join your client machine to your domain and type the UNC path of your member server and drag the certificate to your client Desktop, now we have to import the certificate , for that double click on the certificate you can see the red cross sign here, navigate to the ‘install certificate’ tab, select the ‘Local Machine’ radio button, next, click ok, select ‘Place all certificates in the following store’ radio button and click on browse, select ‘trust root certificate and authority’, ok, ok and finish, to verify again double click on your certificate now you can see the red cross sign is no more.

Now we have to configure SSTP VPN, right click on your start menu and click on ‘network configuration’ navigate to the VPN tab and click on it, click the Add ‘plus’ sign and give the ‘VPN Provider’ as windows (built-in) and click on it, give your VPN name, give the server name as you noticed e.g. ‘sys.john.com’, on the ‘VPN Type’ drop down select SSTP and click on save, now VPN adapter will be created, right click on the adapter and click on ‘connect’, it will ask for credentials give the credentials for your user which you have given permission to remote access and connect it.

Verify: Open command prompt and try to ping DC machine or you can type the UNC path for the share files, you can also right click on the VPN adapter and click on properties and navigate to the ‘details’ tab on the top where you can see the type of the VPN.