DHCP-Configuration

Install DHCP from Add Roles and Features and promote it. Click on tools and open DHCP expand your domain name e.g. john.com and right click on ipv4 and create a new scope name the scope and give the starting and ending ip address including the subnet mask. You can give the excluded address starting and end ip-addresses add options like Router address, DNS address etc, and create the scope.

Verify: Go to your member server logon as administrator remember the adapter should be in internal network no need to change it, only you have to change from static to Dynamic, type ncpa.cpl click on adapter and double click on ipv4 and select the radio buttons to Dynamic for both ip address and DNS address. Disable and enable the adapter, open command prompt and type ‘ipconfig /release’ then ‘ipconfig /renew’ now you are able to see the ip address by typing ‘ipconfig /all’.

Reservation: Right click on the Reservation and click on new reservation click the DHCP radio button give the name of the reservation to remember give the ip address you want to reserve from the pool and the mac-address of the particular machine, in DC machine command prompt type ‘arp –A’ to see the Mac address of the member server copy it by pressing crtl+c and paste it to the Mac address bar and hit ADD.

Verify: Go to member server open command prompt and release then renew the ip address you can see the reservation address is preferred; you can also go to the reservation on DC and refresh to see the lease address as well as reservation address.

NOTE: You can also type ‘getmac’ to see the Mac address of any device.